IoT Lecture Quiz

Weekly Quiz Compiled Study Guide

Lecture Quiz: 2 April 2026, 9:15–10:00 AM  |  50 Questions  |  Closed Book

~60% of questions will be from these weekly quizzes

QUICK STATS

| Total Questions: 159 | Total Topics: 11 |
| --- | --- |

# Week 1: IoT Fundamentals

Q1. The various factors (range, power, data rate, scalability, security and cost) have the same level of importance in any IoT solution. [True/False]

True

✓ False

Answer: False

Q2. Which of the following technology uses 2.4GHz frequency? (Select all that apply) [Multiple answers]

✓ A. Bluetooth/BLE

B. Z-Wave

✓ C. ZigBee

D. Weightless

✓ E. Wireless HART

✓ F. WiFi

G. SigFox

H. NB-IoT

✓ I. LoRa

J. 2G/3G

Answer: Bluetooth/BLE, ZigBee, Wireless HART, WiFi, LoRa

Q3. IoT refers to ... (select the best answer)

A. Interconnections of devices in a network

B. Exchange of data between devices in a network

✓ C. All of them

D. An ecosystem of devices, accessible through internet

Answer: All of them

Q4. Which choice represents the ultimate aim for IoT?

A. To connect as many 'things' together

✓ B. To transform raw data into insights

C. To collect as much data as possible

D. To reduce network bandwidth

Answer: To transform raw data into insights

Q5. Which of the following allows a bunch of heterogeneous systems to communicate and interact with each other?

A. Operating System

B. Hardware

C. Application

✓ D. Middleware

Answer: Middleware

Q6. Which of the following supports the longest range?

A. WiFi

B. ZigBee

C. BLE

✓ D. LoRa

Answer: LoRa

Q7. Which of the following is NOT a component in an IoT device?

A. Controller

B. Sensor

✓ C. Power Supply

D. Communication

E. Actuator

Answer: Power Supply

Q8. Devices in IoT support ... (select the best answer)

A. Actuating

B. Data collection

C. Sensing

✓ D. All of them

Answer: All of them

# Week 2: RESTful APIs

Q1. What does REST stand for?

A. Relative State Transmission

B. Real-time State Transfer

C. Remote Server Transaction

✓ D. Representational State Transfer

Answer: Representational State Transfer

Q2. What type of architecture does RESTful APIs follow?

✓ A. Client-server architecture

B. Monolithic architecture

C. Microservices architecture

D. Peer-to-peer architecture

E. Publisher-Subscriber architecture

Answer: Client-server architecture

Q3. What does idempotence mean in the context of RESTful APIs?

A. The API produces different results with each request

B. The API encrypts data for security

✓ C. The API produces the same result no matter how many times the same request is made

D. The API can handle asynchronous requests

Answer: The API produces the same result no matter how many times the same request is made

Q4. What mechanism does RESTful APIs use to indicate the current state of the resource?

A. HTTP headers

✓ B. HTTP status codes

C. API tokens

D. Response body

Answer: HTTP status codes

Q5. Which of the following is NOT a constraint of RESTful architecture?

A. Client-server architecture

✓ B. Cookie-based authentication

C. Cacheability

D. Statelessness

Answer: Cookie-based authentication

Q6. Which HTTP status code indicates a successful GET request in a RESTful API?

A. 201 Created

B. 500 Internal Server Error

C. 400 Bad Request

✓ D. 200 OK

Answer: 200 OK

Q7. In RESTful APIs, what does a status code in the 5xx range signify?

A. Client error

✓ B. Server error

C. Redirection

D. Successful operation

Answer: Server error

Q8. What is the HTTP method used to create a new resource in RESTful architecture?

✓ A. POST

B. GET

C. PUT

D. DELETE

Answer: POST

Q9. What is a resource in the context of RESTful web services?

A. A set of procedures for accessing a database

✓ B. An entity that can be accessed and manipulated through a URI

C. A function that performs a specific action

D. A container for storing data in a web application

Answer: An entity that can be accessed and manipulated through a URI

Q10. Which of the following HTTP methods is used to retrieve a resource in RESTful architecture?

✓ A. GET

B. DELETE

C. PUT

D. POST

Answer: GET

Q11. What is the primary role of a RESTful API's Uniform Interface constraint?

✓ A. To standardize the way clients interact with the server

B. To separate client and server implementations

C. To define specific operations like GET and POST

D. To ensure that the API is stateful

Answer: To standardize the way clients interact with the server

Q12. Which HTTP method should be used to update an existing resource in RESTful web services?

A. POST

B. GET

C. DELETE

✓ D. PUT

Answer: PUT

Q13. In RESTful APIs, what is the purpose of a payload in a POST request?

A. To carry control information

B. To provide additional headers

✓ C. To contain the data being sent by the client

D. To deliver the client's request to the server

Answer: To contain the data being sent by the client

Q14. In RESTful APIs, what is used to uniquely identify a resource?

A. API key

✓ B. URI (Uniform Resource Identifier)

C. Query parameters

D. Session ID

Answer: URI (Uniform Resource Identifier)

# Week 3: CoAP & MQTT

Q1. What does CoAP stand for?

A. Control Application Protocol

B. Compact Application Protocol

C. Common Application Protocol

✓ D. Constrained Application Protocol

Answer: Constrained Application Protocol

Q2. Which layer is CoAP?

A. Service layer

B. Control layer

✓ C. Application layer

D. Transport layer

Answer: Application layer

Q3. Which protocol is CoAP closely modeled after?

A. MQTT

B. FTP

✓ C. HTTP

D. AMQP

Answer: HTTP

Q4. What transport layer protocol does CoAP use?

A. REST

✓ B. UDP

C. ICMP

D. TCP

Answer: UDP

Q5. CoAP is specialized in?

A. Wireless applications

B. Wired applications

✓ C. Internet applications

D. Device applications

Answer: Internet applications

Q6. CoAP Protocol is comprised of which sub-layers? [Multiple answers]

A. Observation

B. UDP

✓ C. Message

✓ D. Request & Response

Answer: Message, Request & Response

Q7. CoAP supports which of the following methods similar to HTTP? [Multiple answers]

✓ A. GET

✓ B. POST

✓ C. PUT

✓ D. DELETE

E. OPTIONS

F. HEAD

G. CONNECT

H. TRACE

I. PATCH

Answer: GET, POST, PUT, DELETE

Q8. CoAP is a specialised _________ protocol.

A. Web transfer

B. Power

C. Resource

✓ D. Application

Answer: Application

Q9. CoAP messages can be of which types?

A. Request and Response

B. Synchronous and Asynchronous

C. Persistent and Transient

✓ D. Confirmable and Non-Confirmable

Answer: Confirmable and Non-Confirmable

Q10. CoAP provides which of the following requirements?

A. Simplicity and low overhead

B. Multicast support

C. Lower overhead and simplicity

D. Multicast support and simplicity

E. Lower overhead and multicast support

Answer: Multicast support, lower overhead and simplicity

Q11. The core of the CoAP protocol is specified in ______?

✓ A. RFC 7252

B. RFC 7452

C. RFC 7524

D. RFC 7254

Answer: RFC 7252

Q12. What type of protocol is MQTT?

A. Transport protocol

B. Network protocol

C. Presentation protocol

✓ D. Application protocol

Answer: Application protocol

Q13. MQTT is _______ oriented.

A. Data

✓ B. Message

C. Device

D. Network

Answer: Message

Q14. The Last Will Message in MQTT is used to:

✓ A. Notify clients of a failure in the publisher

B. Guarantee that the message was delivered

C. Notify a publisher of a network failure

D. Gain inheritance

Answer: Notify clients of a failure in the publisher

Q15. MQTT uses the publish/subscribe model. How does the publisher know who is subscribing?

A. Publisher uses the 'Eye of Sauron'

✓ B. The publisher doesn't know

C. The publisher gets notified when they subscribe

D. The subscriber informs the publisher

Answer: The publisher doesn't know

Q16. Which of the following is not a client type in MQTT?

A. Publisher

✓ B. Producer

C. Subscriber

D. Broker

Answer: Producer

Q17. Which of the following QoS levels in MQTT ensures message delivery at least once?

✓ A. QoS 1

B. QoS 0

C. QoS 2

D. None of them

Answer: QoS 1

Q18. Which of the following is not a characteristic of MQTT?

A. Quality of Service (QoS)

B. Low overhead

C. Publish/subscribe messaging model

✓ D. Request/response messaging model

Answer: Request/response messaging model

# Week 4: Mesh Networking

Q1. What is the primary feature of a wireless mesh network?

A. Centralized architecture

✓ B. Decentralized architecture

C. Limited to indoor use only

D. Single-hop communication

Answer: Decentralized architecture

Q2. What is a disadvantage of simple flooding in mesh networks?

A. Maximum use of battery power

✓ B. It generates a large number of redundant messages

C. It is too simple to implement

D. Lack of data security

Answer: It generates a large number of redundant messages

Q3. Which flooding algorithm divides the network into subnets for routing?

A. Selective Flooding

B. Simple Flooding

✓ C. Reduced Flooding

D. Adaptive Flooding

Answer: Reduced Flooding

Q4. What does Adaptive Flooding adjust based on?

A. Network conditions and resource availability

B. Static configurations

✓ C. Number of connected nodes

D. The time of day

Answer: Number of connected nodes

Q5. In a distributed mesh network, how are routes between nodes established?

A. All communication goes through a master node

✓ B. Nodes communicate to determine adjacency and routes

C. Through a central authority

D. Using a single, static route

Answer: Nodes communicate to determine adjacency and routes

Q6. What is a typical method to prevent malicious nodes from joining a wireless mesh network?

✓ A. Authentication

B. Ignoring new nodes

C. Decreasing signal strength

D. Redundancy

Answer: Authentication

Q7. Which characteristic is not a feature of wireless mesh networking?

A. Efficient data routing

B. Scalability

C. Self-healing

✓ D. Centralized architecture

Answer: Centralized architecture

Q8. Which is not a challenge mentioned for wireless mesh networks?

A. Interference and reliability

B. Bandwidth allocation

C. Timing and synchronization

✓ D. Unlimited bandwidth

Answer: Unlimited bandwidth

Q9. In the context of mesh networks, what does the term 'self-healing' refer to?

A. The ability to manually repair physical damage

✓ B. The network's ability to automatically reroute data if a node fails

C. The process of adding new nodes to the network

D. The use of hardware issues without human intervention

Answer: The network's ability to automatically reroute data if a node fails

Q10. Dynamic Source Routing have multiple routes per destination. [True/False]

✓ True

False

Answer: True

Q11. Optimized Link State Routing Protocol uses triggered updates of their routing table whenever there is a topology change. [True/False]

True

✓ False

Answer: False

Q12. The Optimized Link State Routing (OLSR) protocol uses ______ to optimize transmission.

A. Dual point relays

B. Single point relays

✓ C. Multipoint relays

D. No relays

Answer: Multipoint relays

# Week 5: LoRa & LoRaWAN

Q1. What does LoRa stand for?

A. Low Range

✓ B. Long Range

C. Long Radio

D. Long Resonance

Answer: Long Range

Q2. What does LoRaWAN stand for?

A. Low Range Wide Area Network

B. Low Radiation Wireless Access Network

C. Logical Routing WAN

✓ D. Long Range Wide Area Network

Answer: Long Range Wide Area Network

Q3. What type of modulation is used in LoRa?

✓ A. Chirp Spread Spectrum (CSS)

B. Phase-Shift Keying (PSK)

C. Frequency Shift Keying (FSK)

D. Amplitude Modulation (AM)

Answer: Chirp Spread Spectrum (CSS)

Q4. LoRa modulation a variation of .....

A. Chirp Spectrum Sector

B. Chord Spread Spectrum

C. Chord Space Spectrum

✓ D. Chirp Spread Spectrum

Answer: Chirp Spread Spectrum

Q5. LoRa is primarily used for?

✓ A. Long-range, low-power communication

B. High-speed data transmission

C. Audio and video transmission

D. Short-range communication

Answer: Long-range, low-power communication

Q6. What is the main drawback of using LoRa for IoT applications?

✓ A. Limited data rate

B. Short range

C. Long range

D. High power consumption

Answer: Limited data rate

Q7. What is the main advantages of using LoRa for IoT applications? (Select all that apply) [Multiple answers]

A. High data rate

✓ B. Low power consumption

✓ C. Long range

D. Low data rate

Answer: Low power consumption, Long range

Q8. In LoRa, what does increasing the spreading factor (SF) do to the signal's range and data rate?

A. Decreases range and increases data rate

✓ B. Increases range and decreases data rate

C. Increases range and data rate

D. Decreases both range and data rate

Answer: Increases range and decreases data rate

Q9. LoRa's spreading factor affects which of the following?

A. Power consumption only

B. Modulation technique

C. Operating frequency

✓ D. Data rate and range

Answer: Data rate and range

Q10. Which Sub-GHz ISM frequency does LoRa operate in Singapore?

A. 2.4 GHz

✓ B. 923 MHz

C. 868 MHz

D. 466 MHz

Answer: 923 MHz

Q11. LoRa modulation technique sits at which OSI layer?

A. Application

B. Session

C. Datalink

D. Network

E. Transport

F. Presentation

✓ G. Physical

Answer: Physical

Q12. LoRaWAN modulation technique sits at which OSI layer?

A. Application

B. Session

✓ C. Datalink

D. Network

E. Transport

F. Presentation

G. Physical

Answer: Datalink

Q13. What does the main function of the Device EUI in LoRaWAN?

A. To define the device's operating frequency

B. To encrypt data

C. To specify the data rate

✓ D. To identify the device uniquely on the network

Answer: To identify the device uniquely on the network

Q14. Which class of LoRaWAN is best suited for applications requiring minimal latency?

A. Class D

B. Class A

✓ C. Class C

D. Class B

Answer: Class C

Q15. Which class of LoRaWAN is designed for bidirectional end-devices with scheduled receive slots?

A. Class C

✓ B. Class B

C. Class D

D. Class A

Answer: Class B

Q16. In LoRa, which class does a battery powered actuator (energy efficient with latency controlled downlink, slotted communication synchronized with a beacon) fall under?

A. C

B. A

✓ C. B

D. D

Answer: B

Q17. LoRaWAN's network server is responsible for:

A. Directly interacting with end-user applications

B. Controlling the output of end devices

C. Managing the radio frequency spectrum

✓ D. Processing and forwarding data to application servers

Answer: Processing and forwarding data to application servers

Q18. Link Budget refers to:

A. The amount of loss that a data link should have

B. The difference between minimum received power level and actual receiver's maximum power level

C. A connection between two entities

✓ D. The amount of loss that a data link (transmitter to receiver) can tolerate in order to operate properly

Answer: The amount of loss that a data link (transmitter to receiver) can tolerate in order to operate properly

# Week 6: WiFi & HaLow (802.11)

Q1. WiFi HaLow is also known as?

A. 802.11f

B. 802.11ac

C. 802.11ax

✓ D. 802.11ah

Answer: 802.11ah

Q2. Which standard does HaLow follow for wireless communication?

✓ A. 802.11ah

B. 802.11ac

C. 802.11n

D. 802.11b

Answer: 802.11ah

Q3. What is the operating frequency of HaLow?

A. 900 Hz

✓ B. Sub-1 GHz

C. 2.4 GHz

D. 5 GHz

Answer: Sub-1 GHz

Q4. What is the typical range of HaLow compared to traditional Wi-Fi?

✓ A. Longer range

B. Shorter range

C. Unlimited range

D. Same range

Answer: Longer range

Q5. What is the main advantages of HaLow over traditional Wi-Fi technologies? (Select all that apply) [Multiple answers]

A. Better security

✓ B. Greater range

✓ C. Lower power consumption

D. Faster speeds

Answer: Greater range, Lower power consumption

Q6. All 802.11 devices are compatible with each other. [True/False]

True

✓ False

Answer: False

Q7. How many overlapping WiFi channels are there for 2.4GHz Band?

A. 14

B. 3

C. 12

✓ D. 11

Answer: 11

Q8. Which if the following channel access technique is used in HaLow?

A. CSMA with Collision Anticipation

✓ B. CSMA with Collision Avoidance

C. CSMA with Collision Prevention

D. CSMA with Collision Detection

Answer: CSMA with Collision Avoidance

Q9. Which of the following power management modes are available for stations in 802.11 networks? (Select all that apply) [Multiple answers]

✓ A. Power Save Mode

B. Doze Mode

C. Awake Mode

✓ D. Active Mode

Answer: Power Save Mode, Active Mode

Q10. Which of the following is included in the beacon to notify stations about the buffered frames?

A. Traffic Intimation Map

B. Traffic Notification Map

✓ C. Traffic Indication Map

D. Traffic Availability Map

Answer: Traffic Indication Map

Q11. Select the standards that are typically common across IEEE 802. (Select all that apply) [Multiple answers]

✓ A. 802.1 Bridging

B. 802.3 Ethernet

C. 802.16 WiMAX

D. 802.17 Resilient Packet Ring

✓ E. 802.2 Logical Link Control

Answer: 802.1 Bridging, 802.2 Logical Link Control

Q12. Which of the following packets help in solving the hidden node problem in 802.11 networks? (Select all that apply) [Multiple answers]

✓ A. Clear To Send Packet

✓ B. Request To Send Packet

C. Clear to Share Packet

D. Request to Share Packet

E. Acknowledgment Packets

Answer: Clear To Send Packet, Request To Send Packet

Q13. The 'Data Unit' gets larger as it goes _____ the OSI stack.

A. up

B. across

C. crisscross

✓ D. down

Answer: down

Q14. Using which parameter will the modulation and coding scheme be decided?

A. Data Transmission Rate

B. None of the options are correct

C. Wireless Communication technique

✓ D. Signal to Noise Ratio

Answer: Signal to Noise Ratio

Q15. Which advantages of using Relays in HaLow? (Select all that apply) [Multiple answers]

✓ A. Relays require devices to transmit at lower data rate consuming low power

B. Relays enable devices to transmit at high data rate

C. Relays allow a connection between two entities

✓ D. Relays improve reliability of data transmission

Answer: Relays require devices to transmit at lower data rate consuming low power, Relays improve reliability of data transmission

Q16. How does HaLow address the shortage of Duration/ID field required for Network Allocation Vector?

A. HaLow uses the Association ID for determining the length of the frame

B. HaLow does not send any mechanism

C. HaLow introduced the Restricted Access Window mechanism

✓ D. HaLow uses a mechanism called Response Indication Deferral

Answer: HaLow uses a mechanism called Response Indication Deferral

Q17. Which frames are transmitted by the Access Point when a node attempts to join a BSS using active scanning? [Multiple answers]

✓ A. Probe Response Frame

B. Association Request Frame

C. Association Response Frame

D. Probe Request Frame

Answer: Probe Response Frame

Q18. Which frames are transmitted by a node when attempting to join a BSS using passive scanning? [Multiple answers]

A. Probe Request Frame

✓ B. Association Request Frame

C. Probe Response Frame

D. Association Response Frame

Answer: Association Request Frame

Q19. Which of the following frames use the Null Data Packet (NDP) MAC frame structure in 802.11ah? [Multiple answers]

✓ A. RTS

B. PS-Poll Frame

✓ C. ACK

D. Beacons

✓ E. CTS

Answer: RTS, ACK, CTS

# Week 7: Bluetooth & BLE

Q1. What does BLE stand for?

A. Bluetooth Long Engagement

B. Bluetooth Low Efficiency

✓ C. Bluetooth Low Energy

D. Basic Link Encryption

Answer: Bluetooth Low Energy

Q2. Which version of Bluetooth introduced BLE?

A. Bluetooth 5.0

✓ B. Bluetooth 4.0

C. Bluetooth 2.0

D. Bluetooth 3.0

Answer: Bluetooth 4.0

Q3. Which of the following supports low energy radio operation?

A. Bluetooth

✓ B. BLE

C. HaLow

D. WiFi

Answer: BLE

Q4. Which frequency band does BLE operate in?

A. 915 MHz

B. 5.0 GHz

C. 433 MHz

D. 868 MHz

✓ E. 2.4 GHz

Answer: 2.4 GHz

Q5. What is the range typically achievable with BLE technology?

✓ A. Up to 100 meters

B. Up to 150 meters

C. Up to 50 meters

D. Up to 10 meters

Answer: Up to 100 meters

Q6. What is the maximum data rate of latest BLE?

A. 4 Mbps

B. 1 Mbps

✓ C. 2 Mbps

D. 3 Mbps

Answer: 2 Mbps

Q7. BLE is particularly designed for:

A. Long-distance communication

B. Audio streaming

✓ C. Short-range, low-power communication

D. High-speed data transfer

Answer: Short-range, low-power communication

Q8. Which feature distinguishes BLE from Classic Bluetooth?

A. Longer range

B. Better audio quality

C. Higher data transfer rates

✓ D. Lower energy consumption

Answer: Lower energy consumption

Q9. Which of the following scheme is used by Bluetooth?

A. Pyramid scheme

✓ B. Frequency hopping TDD scheme

C. DSSS TDD scheme

D. Frequency hopping FDD scheme

E. DSSS FDD scheme

Answer: Frequency hopping TDD scheme

Q10. BLE supports which kind of communication topology? (Select all that apply) [Multiple answers]

✓ A. Point-to-Point

✓ B. Star

C. Bus

✓ D. Mesh

Answer: Point-to-Point, Star, Mesh

Q11. Which of the following Topology does Bluetooth 4.0 support? (Select all that apply) [Multiple answers]

A. Ring

✓ B. Scatter Net

✓ C. Star

D. Mesh

Answer: Scatter Net, Star

Q12. What does 'Pairing' in BLE involve?

A. Connecting two devices for data transfer

✓ B. Exchanging security keys for secure communication

C. Adjusting the power level for optimal connection

D. Broadcasting data to multiple devices

Answer: Exchanging security keys for secure communication

Q13. What is a characteristic of BLE 'Advertising' mode?

A. High data transfer rate

B. Long-range communication

C. Secure data encryption

✓ D. Device discovery and connection establishment

Answer: Device discovery and connection establishment

Q14. Bluetooth is designed for short broadcast. There are forty 2-MHz channels. How many of these channels are reserved for advertising? [Numerical]

✓ A. 3

Answer: 3

Q15. In BLE terminology, what is a 'GATT'?

A. Generic Access Transmission Technique

B. Generic Authentication Transfer Tool

✓ C. Generic Attribute Profile

D. Generic Audio Transmission Technology

Answer: Generic Attribute Profile

Q16. In BLE, what is the role of a 'Central' device?

A. It acts as an intermediary between two devices

B. It broadcasts data to peripheral devices

C. It only receives data

✓ D. It connects to and controls peripheral devices

Answer: It connects to and controls peripheral devices

Q17. Assume Bob can send 300 bits in one slot while using Bluetooth. How many slots does it take to send 600 bits? [Numerical]

A. 2

✓ B. 3

C. 4

Answer: 3

Q18. Assume Bob can send 300 bits in one slot while using Bluetooth. How many slots does it take to send 1200 bits? [Numerical]

A. 4

✓ B. 5

C. 6

Answer: 5

Q19. If the bandwidth of the spread spectrum signal is 180MHz and the signal bandwidth is 15MHz. How many carrier frequencies are required? [Numerical]

✓ A. 12

Answer: 12

Q20. If the number of carrier frequencies is equal to 25, how many output bits must be generated by the PRN generator? (Assume FHSS) [Numerical]

✓ A. 5

Answer: 5

# Week 8: Cloud, Edge & AIoT

Q1. What is Cloud Computing?

✓ A. It is a way to access software and services over the internet

B. It is a way to store data on your local computer

C. It is a way to physically connect multiple computers together

D. It is a way to protect data from unauthorized access

Answer: It is a way to access software and services over the internet

Q2. Which of the following is a benefit of cloud computing?

A. Limited data accessibility

✓ B. Lower maintenance costs

C. Limited scalability

D. Higher upfront costs

Answer: Lower maintenance costs

Q3. What is IaaS?

A. Interaction as a Service

✓ B. Infrastructure as a Service

C. Internet as a Service

D. Intelligence as a Service

Answer: Infrastructure as a Service

Q4. What is the name of the cloud service model that provides access to software applications over the internet?

A. Function as a Service (FaaS)

✓ B. Software as a Service (SaaS)

C. Platform as a Service (PaaS)

D. Infrastructure as a Service (IaaS)

Answer: Software as a Service (SaaS)

Q5. Which if the following is NOT a Cloud Service Model?

A. Infrastructure as a Service

✓ B. Virtualization as a Service

C. Software as a Service

D. Platform as a Service

Answer: Virtualization as a Service

Q6. Which if the following statement is FALSE about Hypervisors?

A. Type 1 Hypervisors are also known as Bare metal hypervisor

B. Type 1 hypervisors do not host OS

C. Type 2 Hypervisors use host OS

✓ D. Type 2 Hypervisors are also known as Bare metal hypervisor

Answer: Type 2 Hypervisors are also known as Bare metal hypervisor

Q7. Which if the following is TRUE about Fog Computing?

A. Fog computing is located nearer to the Cloud computing

✓ B. Cloud is a centralized system, while the Fog is a decentralized system

C. Fog provides low latency

D. Fog computing is a separate architecture that can replace cloud computing

Answer: Cloud is a centralized system, while the Fog is a decentralized system

Q8. Based on the lecture, which is NOT an advantage of Cloud Computing?

A. Resilient

B. Virtualization

C. Management

✓ D. Integration

Answer: Integration

Q9. Which of the following is NOT an advantage of Edge Computing?

A. Faster response time

✓ B. Low investment cost

C. Significantly reduced data storage requirements

D. Reduces Network Bandwidth Requirements

Answer: Low investment cost

Q10. Based on the lecture, which is NOT a typical deployment challenge for Edge computing devices?

A. Thermal

B. Power

✓ C. Sound proofing

D. Waterproofing

Answer: Sound proofing

Q11. AIoT stands for?

A. Artificial Internet of Things

B. Artificial Intelligence of Things

C. Advanced IoT

✓ D. Artificial Intelligence on IoT

Answer: Artificial Intelligence on IoT

Q12. IIoT stands for Industrial IoT. [True/False]

✓ True

False

Answer: True

Q13. What is the purpose of AIoT?

A. To replace IoT devices with AI-powered devices

B. To improve the design of IoT devices

C. To create a secure network for IoT devices

✓ D. To integrate artificial intelligence with the internet of things

Answer: To integrate artificial intelligence with the internet of things

Q14. Which of the following is not a benefit of AIoT? (pick the best answer)

A. Enhanced security

B. Increased productivity

C. Improved energy efficiency

✓ D. Decreased complexity

Answer: Decreased complexity

Q15. Which of the following is not a potential use case for AIoT in industrial automation?

A. Quality control

✓ B. Employee time tracking

C. Inventory management

D. Predictive maintenance

Answer: Employee time tracking

# Week 9: NB-IoT & Cellular IoT

Q1. Which of the following is NOT a mode of operations for NB-IoT?

A. Guard band

B. Stand Alone

C. In-Band

✓ D. Out-Band

Answer: Out-Band

Q2. Which of the following IoT applications is best suited for NB-IoT?

A. Real-time video streaming

B. Wearable fitness trackers

✓ C. Smart metering in rural areas

D. High-speed asset tracking

Answer: Smart metering in rural areas

Q3. Which of the following organizations is responsible for the development of NB-IoT standards?

✓ A. 3GPP

B. IETF

C. Wi-Fi Alliance

D. IEEE

Answer: 3GPP

Q4. Which of the following is a key advantage of NB-IoT over traditional cellular networks?

A. High Data Rates

B. Wide Coverage Area

C. High Reliability

✓ D. Low Power Consumption

Answer: Low Power Consumption

Q5. Why is HTTP not always suitable for IoT devices?

✓ A. It has high overhead and power consumption

B. It lacks support for small data transfers

C. It is incompatible with IoT hardware

D. It is not an IP-based protocol

Answer: It has high overhead and power consumption

Q6. Which of the following features of NB-IoT reduces power consumption by allowing devices to 'sleep' for extended periods?

A. VoLTE

✓ B. PSM

C. eDRX

D. NIDD

Answer: PSM

Q7. Why is NIDD particularly suitable for NB-IoT?

A. It allows direct device-to-device communication

B. It uses a broadcast communication model

C. It supports high-bandwidth applications

✓ D. It eliminates IP overhead, optimizing for small data packets

Answer: It eliminates IP overhead, optimizing for small data packets

Q8. Which network element is responsible for interfacing NIDD with IoT application servers in NB-IoT?

A. MME

B. EPC

C. S-GW

✓ D. SCEF

Answer: SCEF

Q9. Which factors determine the Bit rate of a communication system?

A. The noise level and the power of the transmitted signal

B. The type of modulation and the size of the communication channel

✓ C. The modulation scheme and the symbol rate

D. The carrier frequency and the signal bandwidth

Answer: The modulation scheme and the symbol rate

Q10. Which of the following codes, used in CDMA, are considered orthogonal codes?

A. All of the above

B. <0,0,1,1> and <1,0,1,0>

✓ C. <0,1,0,1> and <1,0,1,0>

D. <0,1,0,1> and <1,0,0,1>

Answer: <0,1,0,1> and <1,0,1,0>

# Week 10: Thread, 802.15.4 & 6LoWPAN

Q1. What does IEEE 802.15.4 primarily focus on?

A. Long-range wireless communication

B. High-bandwidth local area networking

✓ C. Low-rate wireless personal area networks

D. High-speed internet connectivity

Answer: Low-rate wireless personal area networks

Q2. Which type of modulation is used in IEEE 802.15.4?

✓ A. Direct Sequence Spread Spectrum (DSSS)

B. Phase-Shift Keying (PSK)

C. Orthogonal Frequency-Division Multiplexing (OFDM)

D. Frequency Shift Keying (FSK)

Answer: Direct Sequence Spread Spectrum (DSSS)

Q3. IEEE 802.15.4 defines the operation of which network layers? (Select all that apply) [Multiple answers]

A. Session Layer

B. Transport Layer

C. Application Layer

✓ D. Physical (PHY)

✓ E. Data Link Layer

F. Network Layer

Answer: Physical (PHY), Data Link Layer

Q4. IEEE 802.15.4 is a basis for which popular IoT communication protocol? (Select all that apply) [Multiple answers]

A. LTE

B. Bluetooth

C. WiFi

✓ D. Thread

✓ E. ZigBee

Answer: Thread, ZigBee

Q5. 6LowPAN technology sits on top of ...

A. IEEE 802.15.16

B. Anything within IEEE 802

✓ C. IEEE 802.15.4

D. IEEE 802.15.1

Answer: IEEE 802.15.4

Q6. The full form of 6LoWPAN is ...

A. 6 Low Power Wireless Personal Area Network

B. IPv6 over Low-Power Wide Area Networks

✓ C. IPv6 over Low-Power Wireless Personal Area Networks

D. 6 Low Power Wide Personal Area Network

Answer: IPv6 over Low-Power Wireless Personal Area Networks

Q7. 6LowPAN is the same as IEEE 802.15.4. [True/False]

True

✓ False

Answer: False

Q8. 6LowPAN uses traditional IPv4 as its networking layer. [True/False]

True

✓ False

Answer: False

Q9. Which of the following is NOT true about Border Routers in a Thread Network?

A. Border Router provide services to devices within the Thread Network only

B. A REED can be configured as a Border Router

C. Border Router can provide connectivity from Thread Network to 802.11

✓ D. There cannot be more than 3 border routers in a Thread Network

Answer: There cannot be more than 3 border routers in a Thread Network

Q10. Which of the following is NOT a characteristic of Thread?

A. The range of the network is more than enough to cover a home

✓ B. Devices can easily join the network

C. There is no single point of failure

D. Thread can support large networks

Answer: Devices can easily join the network

Q11. Which of the following is true about Thread Network?

A. IPv6 ULA address is used for mesh establishment and link maintenance

B. Every Thread node is assigned its own IPv6 address

C. Thread enables full mesh connectivity between all devices

✓ D. A Thread network can employ more than 10 routers in the network

Answer: A Thread network can employ more than 10 routers in the network

Q12. Which of the following phases are required for a device to re-attach to a Thread Network? (Select all that apply) [Multiple answers]

A. Discovery

B. Commissioning

✓ C. Attaching

D. Configuring

Answer: Attaching

Q13. Which of the following are used by IEEE 802.15.4? (Select all that apply) [Multiple answers]

A. Frequency Hopping Spread Spectrum

✓ B. Direct Sequence Spread Spectrum

✓ C. CSMA with Collision Avoidance

D. CSMA with Collision Detection

Answer: Direct Sequence Spread Spectrum, CSMA with Collision Avoidance

Q14. If the EUI64 address of a device is 62::1, what would be its link local IPv6 address? (Assume 6LoWPAN protocol)

A. FE80:0:0:0:60:1

B. FE80:0:0:0:62::1

C. None of answer mentioned

D. FE80:0:0:0:66::1

✓ E. FE80:0:0:0:22::1

Answer: FE80:0:0:0:22::1

Q15. Assume Bob is using DSSS to transmit his data. He uses chipping sequence 101101001010 (12 bits). Bob transmitted 330 bits total. How many actual data bits does this represent? [Numerical]

✓ A. 30

Answer: 30

Q16. Assume Bob is using DSSS with chipping sequence 101101 (6 bits). If Bob transmitted 100 bits total, how many actual data bits did Bob transmit? [Numerical]

A. ~16-17 bits (600 / 6 = 100 chips)

Answer: 100 bits transmitted = 600 chips; 600/6 = 100 chips

# Week 11: ZigBee & Z-Wave

Q1. Z-Wave is an implementation of a ZigBee protocol. [True/False]

True

✓ False

Answer: False

Q2. What is the purpose of ZigBee communication? (Select all that apply)

A. Healthcare

B. Industrial Control

C. Home Automation

✓ D. All of the above

Answer: All of the above

Q3. What is the role of ZigBee coordinator in the ZigBee network?

A. Controls the network

B. Communicates with other nodes

✓ C. Both

D. None

Answer: Both

Q4. What is the role of ZigBee end devices in the ZigBee network?

✓ A. Communicates with other nodes

B. Controls the network

C. Both

D. None

Answer: Communicates with other nodes

Q5. Which of the following does NOT contribute to routing in a ZigBee network?

✓ A. ZigBee End Device

B. ZigBee Coordinator

C. ZigBee Router

D. ZigBee Master

Answer: ZigBee End Device

Q6. Which network topology is NOT supported by ZigBee?

✓ A. Bus

B. Star

C. Mesh

D. Cluster Tree

Answer: Bus

Q7. Which of the following is needed to be a 'ZigBee Compliant Product'? (Select all that apply) [Multiple answers]

✓ A. IEEE 802.15.4

B. IEEE 802.11

✓ C. Application Profile

✓ D. ZigBee Stack

Answer: IEEE 802.15.4, Application Profile, ZigBee Stack

Q8. Which frequency does Z-Wave operate on?

A. 915 MHz

B. 2.4 GHz

C. 5 GHz

✓ D. 868 MHz

Answer: 868 MHz

Q9. Which of the following is not a feature of ZigBee? (choose the best one)

A. Real-time Communication

✓ B. Large Bandwidth

C. High Security

D. Low Power Consumption

Answer: Large Bandwidth